Chapter Three

3.1 Material Requirement and Selection

i  Material Requirement

Components and Specifications

- Arduino Nano: Microcontroller board with ATmega328P, 5V operation, 32 KB flash memory, 2 KB SRAM, 14 digital I/O pins, 8 analog inputs. Compact size suitable for small enclosures.

- MQ4 Smoke Sensor: Gas sensor sensitive to methane, propane, and butane. Operating voltage: 5V, detection range: 300-10,000 ppm, analog output for smoke concentration.

- Flame Sensor: Infrared-based sensor detecting flame wavelengths (760-1100 nm), 5V operation, analog and digital outputs, detection range up to 1 meter.

- Diode IN4007: General-purpose rectifier diode, 1000V peak reverse voltage, 1A forward current, used for reverse polarity protection.

- DC-DC Buck Converter: Adjustable step-down module, input 6-24V, output 5V/1A to power Arduino and sensors, high efficiency (>90%).

- Transformer: Step-down transformer (220V AC to 12V AC, 500mA) to provide safe input voltage to buck converter.

- ULN2803 Darlington Transistor Array: 8-channel, high-current (500mA per channel), 50V, used to drive the DC buzzer safely.

- DC Buzzer: 5V active buzzer, 85-100 dB, continuous tone for audible alarm.

- Additional Components:

  - Resistors (10kΩ for pull-down, 220Ω for LEDs if used).

  - Capacitors (100µF, 10µF for power stabilization).

  - PCB for prototyping/permanent assembly.

  - Jumper wires, soldering kit, and enclosure (ABS plastic, heat-resistant).

  - Power connector (barrel jack for transformer output).

i  Material Selection

- Compatibility: All components operate at 5V, matching Arduino Nano’s requirements.

- Reliability: MQ4 and flame sensors chosen for high sensitivity and fast response. IN4007 and ULN2803 ensure robust circuit protection and high-current switching.

- Cost-Effectiveness: Components are widely available, affordable, and commonly used in DIY electronics.

- Safety: Transformer and buck converter isolate and regulate power to prevent high-voltage risks. Enclosure protects against environmental factors.

3.2 Construction details

i Construction Procedures

Step 1: Circuit Design and Planning

- Schematic Overview:

  - Arduino Nano as the central controller.

  - MQ4 smoke sensor connected to analog pin (A0) for smoke detection.

  - Flame sensor connected to analog pin (e.g., A1) and digital pin (e.g., D2) for flame detection.

  - ULN2803 drives the DC buzzer, connected to a digital pin (e.g., D3).

  - Transformer output (12V AC) rectified by IN4007, fed to buck converter for 5V DC output.

  - Buzzer activated when smoke or flame exceeds threshold values.

- Tools Required: Soldering iron, multimeter, wire stripper, screwdriver, PCB design software (e.g., Fritzing).

Step 2: Power Supply Assembly

- Connected the transformer’s 220V AC input to a mains plug (ensured proper insulation).

- Attached the transformer’s 12V AC output to a bridge rectifier circuit using four IN4007 diodes.

- Connected the rectified DC output to the buck converter’s input terminals.

- Adjusted the buck converter to output 5V DC, verified with a multimeter.

- Connected the 5V output to Arduino Nano’s VIN pin and GND, and power the sensors via Arduino’s 5V and GND pins.

- Added a 100µF capacitor across the buck converter output for voltage stabilization.

Step 3: Sensor Integration

- MQ4 Smoke Sensor:

  - Connected VCC to Arduino 5V, GND to Arduino GND, and AOUT to Arduino A0.

  - Added a 10kΩ pull-down resistor between AOUT and GND to stabilize readings.

- Flame Sensor:

  - Connected VCC to Arduino 5V, GND to Arduino GND, AOUT to Arduino A1, and DOUT to Arduino D2.

  - Ensured the sensor is positioned to detect flames within its range (adjustable via onboard potentiometer).

Step 4: Buzzer and ULN2803 Setup

- Connected the ULN2803’s input pin (e.g., IN1) to Arduino D3.

- Connected the ULN2803’s corresponding output pin (e.g., OUT1) to the positive terminal of the DC buzzer.

- Connected the buzzer’s negative terminal to ground.

- Powered the ULN2803 via Arduino 5V and GND, ensuring the COM pin is connected to 5V for proper operation.

Fig 3.2 : Mounting of  components on PCB board

Step 5: Programming the Arduino

- Writeed and uploaded the following code to the Arduino Nano using the Arduino IDE:

const int smokePin = A0; // MQ4 analog pin
const int flameAnalogPin = A1; // Flame sensor analog pin
const int flameDigitalPin = 2; // Flame sensor digital pin
const int buzzerPin = 3; // Buzzer control pin via ULN2803
const int smokeThreshold = 300; // Adjust based on calibration
const int flameThreshold = 200; // Adjust based on calibration

void setup() {
pinMode(buzzerPin, OUTPUT);
pinMode(flameDigitalPin, INPUT);
Serial.begin(9600); // For debugging
}

void loop() {
int smokeValue = analogRead(smokePin);
int flameValue = analogRead(flameAnalogPin);
int flameDetected = digitalRead(flameDigitalPin);

Serial.print("Smoke: "); Serial.println(smokeValue);
Serial.print("Flame Analog: "); Serial.println(flameValue);
Serial.print("Flame Digital: "); Serial.println(flameDetected);

// Activate buzzer if smoke or flame detected
if (smokeValue > smokeThreshold || flameValue > flameThreshold || flameDetected == LOW) {
digitalWrite(buzzerPin, HIGH); // Turn on buzzer
} else {
digitalWrite(buzzerPin, LOW); // Turn off buzzer
}

delay(100); // Short delay for stable readings
}

3.2.1 Construction of Casing

. Materials for Casing Construction

- 6x6-inch adaptable white PVC box with a removable cover (water-resistant type is recommended).

- Drill machine with small drill bits (5mm – 20mm) for sensor, LED, and buzzer openings.

Screws and screwdrivers (usually provided with the box).

- Plastic/nylon cable glands or rubber grommets for wiring entry/exit.

- Ventilation mesh or grille for smoke/gas flow into the box.

- Double-sided tape or standoffs for mounting Arduino and sensors inside.

- Heat-resistant epoxy or plastic adhesive for permanent mounting (optional).

Casing Construction Steps

- Box Preparation

Selected a 6x6 adaptable white box with a smooth surface and detachable lid.

Cleaned the box and check that it can **close tightly** to prevent dust and insects from entering.

- Marking Sensor Openings

MQ-4 Smoke Sensor Opening:

Marked a 20mm circular hole on the front or side of the box for smoke access.

Covered the hole with a metal mesh to allow airflow but prevented debris entry.

Flame Sensor Opening:

Marked a 10–12mm round hole facing the area to monitor for flames.

Ensured it is angled downward slightly to detect flame light efficiently.

LED Indicator :

Drilled 5mm hole for LED status indication (power ON or alarm active).

Wiring Access

Marked one hole at the back of the box for the power cable.

Mounting Internal Components

- Arduino Nano & ULN2803 Board

Mounted on the inner back wall using plastic standoffs.

- Buck Converter and Transformer

Mounted at the bottom section to maintain a low center of gravity and prevent wire interference.

Ensure adequate spacing for heat dissipation.

- Sensors

Positioned MQ-4 sensor directly behind its ventilation hole.

Placed the flame sensor behind its dedicated hole, pointing outward.

- Buzzer

Attach near the top hole to allow unobstructed sound.

Fig 3.2 : Mounting of Casing components

Testing After Casing Construction

- Powered the system and check that buzzer sound is clear.

- Verified smoke and flame detection through the new casing.

- Ensured no overheating occurs inside the box due to poor ventilation.

3.3 	Bill of Engineering Measurements and Evaluation

Project: Arduino-based Smoke/Fire Alarm System with 6x6 Adaptable White Box

| S/N | Description of Item | Specification/Size | Qty | Unit | Unit (₦) | Total (₦) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Arduino Nano microcontroller | ATmega328P, 5V | 1 | pcs | 15,000 | 15,000 |
| 2 | MQ-4 Smoke/Gas sensor | 5V, analog output | 1 | pcs | 8,000 | 8,000 |
| 3 | Flame sensor module | 5V, IR-based | 1 | pcs | 5,000 | 5,000 |
| 4 | ULN2803 transistor array | 8-channel, 500mA | 1 | pcs | 2,500 | 2,500 |
| 5 | DC Buzzer | 5V, 85–100 dB | 1 | pcs | 3,000 | 3,000 |
| 6 | 1N4007 Diode | 1A, 1000V | 4 | pcs | 100 | 400 |
| 7 | DC-DC Buck Converter | Input 7–35V → 5V/2A | 1 | pcs | 8,000 | 8,000 |
| 8 | Step-down Transformer | 220V → 12V AC, 1A | 1 | pcs | 15,000 | 15,000 |
| 9 | LEDs (Status Indicator) | 5mm, red/green | 2 | pcs | 100 | 200 |
| 10 | Resistors & Capacitors | For filtering/protection | 1 | set | 100 | 100 |
| 11 | 6x6 Adaptable White Box | PVC, with cover | 1 | pcs | 2,000 | 2,000 |
| 12 | Metal Mesh/Grille | For sensor vent | 1 | pcs | 1,5000 | 1,500 |
| 13 | Nylon Cable Gland/Grommet | 10mm | 1 | pcs | 2,000 | 2,000 |
| 14 | PCB or Breadboard | 7x9 cm | 1 | pcs | 2,000 | 2,000 |
| 15 | Standoffs / Double-sided Tape | Plastic | 1 | set | 500 | 500 |
| 16 | Screws and Nuts | Stainless steel | 1 | set | 500 | 500 |
|  |  | TOTAL |  |  |  | 65,700 |

Chapter Four: Conclusion and Recommendations

4.1	Conclusion

The design and construction of the Arduino-based Smoke/Fire Alarm System using the MQ-4 smoke sensor, flame sensor, Arduino Nano, ULN2803 driver, and 6x6 adaptable white box casing successfully demonstrate an efficient and cost-effective approach to early fire detection.

The system is capable of:

- Detecting smoke and flames accurately through its dual-sensor configuration.

- Providing timely audio alerts using a 5V DC buzzer for immediate response.

- Operating reliably on a stable power supply derived from a transformer and DC-DC buck converter.

- Ensuring safety and durability through a compact, ventilated casing that protects internal electronics while allowing proper gas and flame sensing.

This project highlights the practical application of embedded systems in fire safety, and its scalability allows it to be deployed in residential, commercial, and small industrial environments. Future improvements can include GSM notifications, IoT integration, or battery backup for enhanced functionality.

4.2	Recommendations

Based on the successful design and implementation of the Arduino-based Smoke/Fire Alarm System, the following recommendations are proposed for improved reliability and real-world deployment:

Power Backup Integration:

Incorporate a rechargeable battery or UPS to ensure continuous operation during power outages.

- IoT and Remote Monitoring:

- Integrate GSM or Wi-Fi modules to send SMS or mobile notifications** when smoke or flame is detected.

Multi-Zone Expansion:

Implement multiple sensor nodes for larger buildings, with a central control unit for real-time monitoring.

Environmental Calibration:

Periodically calibrate the MQ-4 sensor to maintain accuracy in various humidity and temperature conditions.

References

1. Arduino Official Documentation. (2025). *Arduino Nano Board Overview*. Retrieved from [https://docs.arduino.cc](https://docs.arduino.cc)

2. Hanwei Electronics Co., Ltd.** (2024). MQ-4 Methane and Natural Gas Sensor Datasheet*. Retrieved from [http://www.hwsensor.com](http://www.hwsensor.com)

3. Flame Sensor Module Datasheet. (2024). *Infrared Flame Detection Module Specifications*. Retrieved from [https://components101.com](https://components101.com)

4. ULN2803A Datashee. (2023). *8-Channel Darlington Transistor Array*. Texas Instruments. Retrieved from [https://www.ti.com](https://www.ti.com)

5.1N4007 Diode Datasheet. (2023). *Standard Recovery Rectifier, 1A 1000V*. Vishay Semiconductors.

6. Petruzella, F. D. (2022). *Electronics and Electrical Systems for Safety Devices*. McGraw-Hill Education.

7. Sharma, R. & Singh, A. (2023). *Design and Implementation of IoT-based Fire Alarm Systems*. International Journal of Embedded Systems, 15(2), 45–53.

8. National Fire Protection Association (NFPA). (2024).NFPA 72: National Fire Alarm and Signaling Code Retrieved from [https://www.nfpa.org](https://www.nfpa.org)

Appendix

Appendix A: Arduino Source Code

// Arduino-based Smoke and Flame Detection System
#define MQ4_PIN A0
#define FLAME_PIN 2
#define BUZZER_PIN 5

int smokeThreshold = 350;
int smokeValue = 0;

void setup() {
  Serial.begin(9600);
  pinMode(FLAME_PIN, INPUT);
  pinMode(BUZZER_PIN, OUTPUT);
}

void loop() {
  smokeValue = analogRead(MQ4_PIN);
  int flameDetected = digitalRead(FLAME_PIN);

  Serial.print("Smoke Value: ");
  Serial.print(smokeValue);
  Serial.print(" | Flame: ");
  Serial.println(flameDetected ? "No" : "Yes");

  if (smokeValue > smokeThreshold || flameDetected == LOW) {
    digitalWrite(BUZZER_PIN, HIGH);
  } else {
    digitalWrite(BUZZER_PIN, LOW);
  }

  delay(500);
}

Appendix B: Pin Connections Table

MQ-4 Sensor (AO) → A0
MQ-4 Sensor (VCC) → 5V
MQ-4 Sensor (GND) → GND
Flame Sensor (DO) → D2
Flame Sensor (VCC) → 5V
Flame Sensor (GND) → GND
Buzzer via ULN2803 → D5
ULN2803 Output → Buzzer +
Buck Converter Output → 5V to Arduino VIN

Appendix C: Testing Procedure

1. Power up the system and verify 5V supply.
2. Observe serial output for baseline smoke readings.
3. Introduce controlled smoke to test MQ-4 activation.
4. Use a lighter flame to test flame sensor without direct contact.
5. Confirm buzzer activation and alarm reset.